Name:

Date:

Investigating Electrical Characteristics

STUDENT

Introduction

In this experiment you will be determining the current – voltage characteristic of an electrical component.

Aim

- To plan an experimental procedure with a clear investigative aspect

- To set up a test circuit correctly

- To obtain an appropriate set of data

- To plot the characteristic I-V graph for several components

Intended class time

- 45 to 60 minutes

Equipment (per group)

- Power supply (max 12V)

- Rheostat/potentiometer

- Ammeter

- Voltmeter

- Leads

- Test components (resistor, filament lamp, diode, LED)

Health and safety

- Safe use of electrical circuits

- Work within the limits of voltage and current given by your teacher

- Do not short the cells out

Task - planning

- Use information on p148 to help you to decide on an appropriate circuit to use in your investigation – explain the reasons for the choice you made.

- Draw the circuit diagram for the circuit you decide to use.

- Note down any safety issues you need to take into account

- To measure the characteristic curve for a component you must change the voltage and measure the current, this should include reversing the polarity of the supply to obtain readings for negative voltage – explain how you are going to do this.

- Set up your test circuit with a filament lamp – test to find out what the range of current and voltage will be and write them down in your plan.

- Draw a data table that will enable you to collect a minimum of 10 pairs of data points	between your minimum and maximum voltage values

Name:

Date:

Investigating Electrical Characteristics

STUDENT

Introduction

In this experiment you will be determining the current – voltage characteristic of an electrical component.

Aim

- To plan an experimental procedure with a clear investigative aspect

- To set up a test circuit correctly

- To obtain an appropriate set of data

- To plot the characteristic I-V graph for several components

Intended class time

- 45 to 60 minutes

Equipment (per group)

- Power supply (max 12V)

- Rheostat/potentiometer

- Ammeter

- Voltmeter

- Leads

- Test components (resistor, filament lamp, diode, LED)

Health and safety

- Safe use of electrical circuits

- Work within the limits of voltage and current given by your teacher

- Do not short the cells out

Task - planning

- Use information on p148 to help you to decide on an appropriate circuit to use in your investigation – explain the reasons for the choice you made.

- Draw the circuit diagram for the circuit you decide to use.

- Note down any safety issues you need to take into account

- To measure the characteristic curve for a component you must change the voltage and measure the current, this should include reversing the polarity of the supply to obtain readings for negative voltage – explain how you are going to do this.

- Set up your test circuit with a filament lamp – test to find out what the range of current and voltage will be and write them down in your plan.

- Draw a data table that will enable you to collect a minimum of 10 pairs of data points	between your minimum and maximum voltage values

Task – obtaining data

- Measure the current as the voltage is changed across the component.

- Draw graphs of the current – voltage curves for your component.

- Calculate the resistance of the component at several points within your data range.

- Describe the characteristic of the component with relation to potential difference, current and resistance.

To submit

For this piece of work to count towards Practical Activity Group 3 of the GCE Physics Practical Endorsement you should have the following evidence:

- Data collected from your group in a clear and logical format – as required by the PSH

- Current-voltage graphs, drawn according to the guidelines in the PSH

- Your description of characteristic current-voltage behaviour for each component

Task – obtaining data

- Measure the current as the voltage is changed across the component.

- Draw graphs of the current – voltage curves for your component.

- Calculate the resistance of the component at several points within your data range.

- Describe the characteristic of the component with relation to potential difference, current and resistance.

To submit

For this piece of work to count towards Practical Activity Group 3 of the GCE Physics Practical Endorsement you should have the following evidence:

- Data collected from your group in a clear and logical format – as required by the PSH

- Current-voltage graphs, drawn according to the guidelines in the PSH

- Your description of characteristic current-voltage behaviour for each component